Reduced footprint SDV for PCB Microfactory

\author{\IEEEauthorblockN{1\textsuperscript{st} Alejandro Ojeda Olarte}

\IEEEauthorblockA{\textit{Mechanical and Mechatronics Engineering Department} \\

\textit{Universidad Nacional de Colombia}\\

Bogotá, Colombia \\

aojedao@unal.edu.co}

\and

\IEEEauthorblockN{2\textsuperscript{nd} Nicolás Pulido Gerena}

\IEEEauthorblockA{\textit{Mechanical and Mechatronics Engineering Department} \\

\textit{Universidad Nacional de Colombia}\\

Bogotá, Colombia \\

npulido@unal.edu.co}

\and

\IEEEauthorblockN{3\textsuperscript{rd} Ernesto Córdoba Nieto}

\IEEEauthorblockA{\textit{Mechanical and Mechatronics Engineering Department} \\

\textit{Universidad Nacional de Colombia}\\

Bogotá, Colombia \\

ecordoban@unal.edu.co}

\and

}

\maketitle

\begin{abstract}

This document discusses the design process of a differential SDV with 2D perception for a PCB microfactory using flexible operation processes. Using a Raspberry Pi 3B and a BeagleBone Blue, a $5.02 \, kg$ payload-carrying robot is designed and manufactured. Perception is achieved using a RPLidar A1 and Hector SLAM mapping.

\end{abstract}

\begin{IEEEkeywords}

Robot, Differential Drive, Hector SLAM, Microfactory, PCB, SDV

\end{IEEEkeywords}

\section{Introduction}

Microfactories, are small scale production plants that help increase manufacturing flexibility in small production batches. Given to the fact micro factories use smaller areas than traditional factories, the internal products transport system requires minimizing this space as much as possible.

\subsection{Industry 4.0}\label{AA}

Industry 4.0 has established connectivity and collaborative robots in the manufacturing processes, enabling work areas where humans and robots work in safe enviroments and share information to optimize workflows \cite{shneier2015mobilerobotsmanufacturing}.

Mobile robots and manipulators enchances flexible and autonomous automation, which is required to build Smart Factories, whose most important goal is to integrate information shared with new trends like Iot, AI or Big data \cite{messina2002autonomousmobilerobots}.

\subsection{Microfactories}

A microfactory's goal is to reduce the plant's footprint and the energy required to use it. Due to this scale reduction, waste generated by manufacturing each product is reduced as well.  \cite{mishima2002design}.

\subsection{Differential Robot}

Differential robots motion are describes as follows \cite{malu2014kinematics}.

$$\dot x=v \cos{\theta}$$

$$\dot y = v \sin{\theta}$$

$$\dot \theta = \omega$$

And for each $t_k$ time the configuration is described as follows

$$ \begin{bmatrix}

x_k\\

y_k\\

\theta_k

\end{bmatrix} = \begin{bmatrix}

x_{k-1}\\

y_{k-1}\\

\theta_{k-1}

\end{bmatrix} + \begin{bmatrix}

\cos{\theta} & 0 \\

\sin{\theta} & 0 \\

0 & 1

\end{bmatrix} \begin{bmatrix}

\Delta s\\

\Delta \theta

\end{bmatrix}$$

\section{Design}

In order to improve flexibility and connectivity in a Printed Circuit Board micro factory, a self driving vehicle (SDV) was designed. This robot, in comparison with Autonomus Guided Vehicles (AGV), don't required new infrastructure to be able to navigate freely in its work environment.

As well, a replicable robot, that can be manufactured using the Universidad Nacional de Colombia Sede Bogota Experimental Factory Laboratory (LabFabEx) machinery, easy to assemble and small production time, in order to build a transport fleet using this robot as a base model.

The initial proposal was to design a differential configuration for the SDV, as it is shown in figure \ref{fig3_1}. This way, assembly is simplified and flexibilized, and improves mobility in the workspace.

\begin{figure}[htbp]

\centerline{\includegraphics[width=0.3\textwidth]{robotdiferencial.jpeg}}

\caption{Differential Robot \cite{ivan26modelacion}. }

\label{fig3_1}

\end{figure}

The kinematic diagram showed in figure \ref{fig3_2}, describes the model mentioned before, as well as the restrictions imposed by it.

\begin{figure}[htbp]

\centerline{\includegraphics[width=0.3\textwidth]{Configuration-of-a-differential-drive-wheeled-mobile-robot.png}}

\caption{Kinematic Diagram of a Differential Robot \cite{malu2014kinematics}}

\label{fig3_2}

\end{figure}

\subsection{Axiomatic Design}

In matrix A of axiomatic design, the requirements imposed are assigned to the solution proposed in this model. \cite{suh1998axiomatic}\\

\begin{tabular}{|m{5em} | m{4em}| m{5em} |m{5 em} | }

\hline

{} & ROS & Differential Configuration & LIDAR\\

\hline

Self Driving & x & 0 & 0 \\

\hline

360° Movement & 0 & x & 0  \\

\hline

360° Perception & 0 & 0 & x \\

\hline

\end{tabular}\\

Therefore, matrix B, which relates physical components with the design matrix, outputs the selected devices that will satisfy the robot characteristics.

\begin{tabular}{|m{5em} | m{4em}| m{5em} |m{5 em} | }

\hline

{} & Raspberry Pi 3B & Gearmotor & RPLidar A1\\

\hline

ROS & x & 0 & 0 \\

\hline

Differential Conf. & 0 & x & 0  \\

\hline

RPLidar A1 & 0 & 0 & x \\

\hline

\end{tabular}

\subsection{Key Performance Indicators}

\begin{itemize}

\item Size: The robot occupies $ 0.49 \, m^2$, and the chassis diameter is $14 \,cm$.

\item Payload Capacity: The robot is able to transport up to a $5.02 kg$ payload.

\item Power Source: It uses two power sources, a $2S$ Lipo battery ($7.4V$) and a commercial $5V$ Powerbank.

\item Components Quantity: The robot requires $10$ different component types, two control boards, motors, spacers, two chassis platforms, two PCB clips, two batteries and wheels.

\end{itemize}

A radar chart of these indicators are showed in figure \ref{fig1}. \\

\begin{figure}[htbp]

\centerline{\includegraphics[width=0.5\textwidth]{KPI Value.png}}

\caption{KPIs radar chart.}

\label{fig1}

\end{figure}

\subsection{Electronic Design}

There are two control boards, one that will carry out the high-complexity tasks, and another that will carry out the low-complexity ones.

\begin{itemize}

\item BeagleBone Blue: this board's microprocessor is an Octavo OSD3358, and has a $512MB$ DD3 RAM memory at $1GHz$ .  It runs a modified Ubuntu MATE 18 (Bionic Beaver) version, designed by Robert C. Nelson, which includes the $librobotcontrol$ libary. This library is a hardware interface for the Beagle Bone Blue, which counts with $4$ $qwiic$ connectors for encoders,dual TB6612FNG drivers for DC motors, 9 axis IMU , multiple SPI ports, UART ports and 8 servomotor outputs. The board also counts with a 2S LIPO ($7.4V$) balance connector.

Due to the integrated hardware, this board will carry out the low-complexity tasks, therefore will act as the slave board.

\item Raspberry Pi 3B: The $1GB$ RAM model was selected to run the high-complexity tasks, among them are running the $ROSMASTER$ node, which connects to the BeagleBone Blue trough USB internet virtual adapter,and processes the RPLidar A1 information. In order to do that, the LIDAR uses the SLAMTEC node and publishes the $/scan$ topic. Afterwards, all the Hector SLAM nodes are run in this board as well. It runs a modified Debian Buster with ROS Melodic.

As well, to visualize the RViz generated map, the Raspberry runs a VNC server.

\end{itemize}

These cards are controlled through a computer that connects to the Raspberry through the SSH protocol (Secure Shell Protocol) and through the VNC server, both via WLAN using card's IP address. Then, the BeagleBone is accessed via SSH using the USB local network, which by default corresponds to the $192.168.7.2$ address, whereas the Raspberry will appear as $192.168.7.1$ IP. This last address was configured as ROS\_HOSTNAME. The computer was the intermediary to use the interaction interfaces for remote navigation.

\subsection{Mechanical Design}

\begin{itemize}

\item Chassis: It was designed in two levels, superior and inferior levels. The inferior level is supported in a flat base which can be manufactured in different methods and materials, to achieve operation flexibility.

\begin{itemize}

\item Flat bases:

\begin{itemize}

\item CNC Manufacturing: MDF, Acrylic, Aluminum

\item Additive Manufacturing: PLA

\item Laser Cutting: MDF, Acrylic

\end{itemize}

\item Motor bases:

\begin{itemize}

\item CNC Manufacturing: MDF, Aluminum

\item Additive Manufacturing: PLA, SLA

\end{itemize}

\item Spacers:

\begin{itemize}

\item Additive Manufacturing: PLA, SLA

\end{itemize}

\item PCB Clip:

\begin{itemize}

\item CNC Manufacturing: MDF, Aluminum

\item Additive Manufacturing: PLA, SLA

\end{itemize}

\item RPLidar adapter:

\begin{itemize}

\item CNC Manufacturing: MDF, Aluminum

\item Additive Manufacturing: PLA, SLA

\end{itemize}

\end{itemize}

The CAD model designed is shown in figure \ref{fig6}.

\begin{figure}[htbp]

\centerline{\includegraphics[width=0.5\textwidth]{MicroRobot v17.png}}

\caption{Robot CAD model.}

\label{fig6}

\end{figure}

\item Motors: Two Pololu Metal Gearmotors were selected. They have a 78:1 reduction ratio, operate on 6V and count with an extended motor shaft.

\end{itemize}

\subsection{Additive Manufacturing}

The PCB Clips were manufactured using Fused Filament Fabrication (FFF). Two different clip designs were modeled, and the one on the right in figure \ref{fig2} demonstrated better rigidity to hold the FR4 board.

\begin{figure}[htbp]

\centerline{\includegraphics[width=0.28\textwidth]{Clips.png}}

\caption{Diseño de Clips propuestos}

\label{fig2}

\end{figure}

It was printed using the LabFabEx UN FDM 3D Printer in PLA, the results are shown in figure \ref{fig2_1}.

\begin{figure}[htbp]

\centerline{\includegraphics[width=0.28\textwidth]{Clips impresos.png}}

\caption{Printed PCB Clips}

\label{fig2_1}

\end{figure}

\subsection{Control Design}

Both boards run ROS Melodic, the BeagleBone Blue runs 2 nodes and the Raspberry Pi run 3, as well as the master node. The system architecture is then obtained using the RQt ROS user interface tools, it is shiwn in figure \ref{fig2_2}.

\begin{figure}[htbp]

\centerline{\includegraphics[width=0.5\textwidth]{RobotSLAM.png}}

\caption{RQt Graph}

\label{fig2_2}

\end{figure}

\subsection{Communications Design}

The communications diagram is showed in figure \ref{fig2_3}

\begin{figure}[htbp]

\centerline{\includegraphics[width=0.5\textwidth]{ROSDiagramNetwork.png}}

\caption{Communications Diagram}

\label{fig2_3}

\end{figure}

The Ethernet protocol that runs between the Raspberry and the BeagleBone Blue uses the USB port of the BeagleBone Blue, while the connection between the Raspberry Pi and the user computer is via WLAN.

\subsection{Robot Characteristics}

The Robot is able to transport $5.2 kg$ payloads.

At a nominal $0.5 \frac{m}{s}$ speed, the effective speed is $0.34 \frac{m}{s}$. For a $0.257 \frac{m}{s}$ nominal speed, the effective speed is $0.18 \frac{m}{s}$

\subsection{Process Oriented Analysis Diagram}

In order to have an optimal design for industrial production in the proposed microfactory, the following POA (Process Oriented Analysis) diagram was designed. There, the two internal steps that the mobile robot performs to function internally and the steps that it must perform in its environment to complete the PCB manufacturing process are shown in figure \ref{fig3}

\begin{figure}[htbp]

\centerline{\includegraphics[width=0.5\textwidth]{POA.png}}

\caption{POA Diagram}

\label{fig3}

\end{figure}

\subsection{Navigation}

The robot performs SLAM by means of the Hector SLAM algorithm. Position feedback is done by analysis of the BeagleBone Blue's 9-axis IMU. A transformation to the original point using the tf node is required. Figure \ref{fig5} shows the algorithm running in a controlled, closed area for testing.

\begin{figure}[htbp]

\centerline{\includegraphics[width=0.5\textwidth]{slam.png}}

\caption{Hector SLAM running in the robot.}

\label{fig5}

\end{figure}

\section{Conclusions}

For a microfactory, a differential robot configuration reduces the space required by a robot for maneuvering. Likewise, a 360° LIDAR facilitates perception in small areas. The real robot is shown in figure \ref{fig7}. Future versions of this robot can be integrated in the LabFabEx UN web platform, as well as be integrated in a fleet of multiple SDVs.

\begin{figure}[htbp]

\centerline{\includegraphics[width=0.3\textwidth]{MicrobotReal.png}}

\caption{Completed robot.}

\label{fig7}

\end{figure}

\begin{thebibliography}{00}

\bibitem{shneier2015mobilerobotsmanufacturing}Shneier, M. \& Bostelman, R. Literature Review of Mobile Robots for Manufacturing. {\em National Institute of Standards Technology Technical Series Publications}. (2015)

\bibitem{messina2002autonomousmobilerobots}Messina, E. Performance evaluation of autonomous mobile robots. {\em Industrial Robot Journal}. (2002)

\bibitem{mishima2002design}Mishima, N., Tanikawa, T., Ashida, K. \& Maekawa, H. Design of a Microfactory. {\em International Design Engineering Technical Conferences And Computers And Information In Engineering Conference}. \textbf{36231} pp. 103-110 (2002)

\bibitem{malu2014kinematics}Malu, S. \& Majumdar, J. Kinematics, localization and control of differential drive mobile robot. {\em Global Journal Of Research In Engineering}. (2014)

\bibitem{suh1998axiomatic}Suh, N. Axiomatic design theory for systems. {\em Research In Engineering Design}. \textbf{10}, 189-209 (1998)

\bibitem{clauer2021axiomaticmobilerobots_1}Clauer, D., Fottner, J., Rauch, E. \& Prüglmeier, M. Usage of Autonomous Mobile Robots Outdoors – an Axiomatic Design Approach {\em CIRPe 2020 – 8th CIRP Global Web Conference – Flexible Mass Customisation}. (2021)

\bibitem{clauer2021axiomaticmobilerobots_2}Marx, W., Mavris, D. \& Schrage, D. Integrating design and manufacturing for a high speed civil transport wing {\em Georgia Institute of Technology, School of Aerospace Engineering}. (2000)

\bibitem{shen2016sla}Shen, J., Aiken, M., Ladd, C., Dickey, M. \& Ricketts, D. A Simple Electroless Plating Solution for 3D Printed Microwave Components {\em Asia-Pacific Microwave Conference}. (2016)

\bibitem{jones2017sla}Jones, C., Mills, B., Nishimoto, R. \& Robinson, D. Electroless Deposition of Palladium on Macroscopic 3D-Printed Polymers with Dense Microlattice Architectures for Development of Multifunctional Composite Materials {\em Journal of The Electrochemical Society}. (2017)

\bibitem{kolczyk2018sla}Kołczyk, K., Zborowski, W., Kutyła, D., Kwiecińska, A., Kowalik, R. \& Żabiński, P. Investigation of two-step metallization process of plastic 3d prints fabricated by SLA method {\em Arch. Metall. Mater}. (2018)

\bibitem{huang2020sla}Huang, J., Qin, Q. \& Wang, J. A Review of Stereolithography: Processes and Systems {\em School of Mechanical Engineering, Sichuan University}. (2020)

\bibitem{choo2022sla}Choo, S., Jin, S. \& Jung, J. Fabricating High-Resolution and High-Dimensional Microneedle Mold through the Resolution Improvement of Stereolithography 3D Printing {\em Department of Pharmaceutical Engineering, Dankook University}. (2022)

\bibitem{gaspar2008metallicsla}Gaspar, J. \& Bártolo, P. Metallic Stereolithography {\em 9th Biennial ASME Conference on Engineering Systems Design and Analysis}. (2008)

\bibitem{szajowicz2018flexible}Szajowicz, D., Colorado, J. \& Mondragon, I. Indoor mapping using SLAM for applications in Flexible Manufacturing Systems {\em Universidad Javeriana}. (2018)

\bibitem{chang2016flexible}Chang, Y. \& Finkbeiner, M. Evaluating sustainable development from a child’s perspective - a proposal of Sustainable Child Development Index (SCDI) {\em 13th Global Conference on Sustainable Manufacturing}. (2016)

\bibitem{teso-fz-betono2019motionplanning}Teso-Fz-Betoño, D., Zulueta, E., Fernandez-Gamiz, U., Aramendia, I. \& Uriarte, I. A Free Navigation of an AGV to a Non-Static Target with Obstacle Avoidance {\em Electronics Journal}. (2019)

\bibitem{gutierrez2020motionplanning}Gutiérrez, R., López-Guillén, E., Bergasa, L., Barea, R., Pérez, O., Gómez-Huélamo, C., Arango, F., del Egido, J. \& López-Fernández, J. A Waypoint Tracking Controller for Autonomous Road Vehicles Using ROS Framework {\em Sensors Journal}. (2020)

\bibitem{haar2017motionplanning}Haar, J., Pisinger, D. \& Sabbaghian, M. A dynamic programming approach for optimizing train speed profiles with speed

restrictions and passage points {\em Transportation Research Part B: Methodological}. (2017)

\bibitem{seder2007motionplanning}Seder, M. \& Petrovic, I. Dynamic window based approach to mobile robot motion control in the presence of moving obstacles {\em IEEE International Conference on Robotics and Automation}. (2007)

\bibitem{yin2015motionplanning}Yin, J., Yang, G., Zhao, F. \& Qiu, H. Motion Planning Implemented in ROS for Omni-directional Wheeled Mobile Robot {\em IEEE International Conference on Information and Automation}. (2015)

\bibitem{albers2019motionplanning}Albers, F., Rösmann, C., Hoffmann, F. \& Bertram, T. Online Trajectory Optimization and Navigation in Dynamic Environments in ROS {\em Studies in Computational Intelligence}. (2019)

\bibitem{oberg2012cnc}Oberg, E., Jones, F., Horton, H. \& Ryffel, H. Machinery’s Handbook. 29th Edition {\em Industrial Press, New York}. (2012)

\bibitem{ivan26modelacion}Iván, C., Efrén, G., Ortega, J., Carrillo, U., Torrez, R. \& Aguirre, M. Modelación de un Robot Móvil de Dos Ruedas con Tracción Diferencial. (8º Congreso Nacional de Mecatrónica, Noviembre)

\end{thebibliography}

\vspace{12pt}

\end{document}